Security of Drones
Objective 2

Note: Teams should provide sufficient technical detail to demonstrate feasibility without submitting generic or excessively long material.

Stage 1 Participant Submission - Report Structure

# Team and Submission Information

| Particular | Details |
| --- | --- |
| Team ID | TM-8AA11E658A0 |
| Team Name | Nakshatra |
| Institution / Organization | CSPIT, CHARUSAT |
| Team Leader | Dhairya Patel |
| Team Members | Astha Patel |
| Faculty / Industry Mentor, if any | Axat Patel |
| Email Address | Dhairyapatel423@gmail.com |
| Contact Number | 9664852763 |
| Proposed Design Name | Fusion-Based Real-Time Drone IDS (FRT-IDS) |
| Date of Submission | 27-09-2026 |

Objective: Drone Intrusion Detection System (Drone IDS)

1. Executive Summary
The Drone IDS is an onboard intrusion detection system for UAVs running ArduPilot on Pixhawk-class flight controllers, designed to run on a companion computer (Raspberry Pi 3B+ / Jetson Nano / equivalent ARM64) alongside a laptop-hosted SITL environment for development and offline evaluation. The system implements a 9-stage pipeline processing MAVLink 2.0 telemetry and command streams in real time, with each stage targeting a distinct attack surface: CRC integrity (Layer 1), anti-replay sliding window (Layer 2), command-injection state-machine validation (Layer 3), RF-jamming statistics from RADIO_STATUS (Layer 4), firmware attack detection via manifest integrity and behavioral fingerprinting (Layer 5–7), control-system cyber-physical fusion (Layer 8), and navigation-sensor cross-check fusion (Layer 9).

The key innovation is multi-tier cyber-physical fusion: each domain (communication, firmware, control-system, navigation) runs independent detectors that emit structured alerts with MITRE ATT&CK for ICS tags, attack-class labels, affected-channel identifiers, and recommended advisory actions (`distrust` / `caution` / `none`). A weighted fusion layer in the control-system module combines six individual detectors (sensor consistency, EKF innovation health with CUSUM/chi², control invariant, actuator anomaly with Goertzel spectral analysis, parameter tampering, failsafe logic) into fused alerts. The navigation module adds 18 detectors (NAV-001…NAV-018) covering GNSS jamming, GPS spoofing (jump, drift, seamless, time), barometer/magnetometer/optical-flow/rangefinder spoofing, RTK correction faults, and coordinated multi-sensor attacks, with per-channel trust scores and hysteresis that drive navigation-source recommendations (`can_re_anchor` flag for dead-reckoning re-anchoring).

Target platforms: ArduPilot Copter 4.4+/4.5+ on Pixhawk 4 / Cube Orange; companion computer Raspberry Pi 3B+ or Jetson Nano (ARM64); development on Windows 11 laptop with SITL (`arducopter -f simple -I0`). Monitored MAVLink messages: HEARTBEAT (mode, custom_mode), GPS_RAW_INT / GPS2_RAW (position, velocity, sats, fix, eph, epv, CN0), GLOBAL_POSITION_INT, SCALED_PRESSURE / SCALED_IMU / HIGHRES_IMU, ATTITUDE, VFR_HUD, EKF_STATUS_REPORT, RADIO_STATUS, COMMAND_LONG, FILE_TRANSFER_PROTOCOL, PARAM_SET, AUTOPILOT_VERSION, SYS_STATUS, ESTIMATOR_STATUS, OPTICAL_FLOW, DISTANCE_SENSOR, SERVO_OUTPUT_RAW, NAV_CONTROLLER_OUTPUT.

Major detection capabilities with headline metrics (offline synthetic replay, 500 benign packets, Windows host):

- **Command Injection**: 12 rules (CRC, replay, unauthorized source, state violation, param bounds, RF jamming) — precision 1.000, recall 1.000, F1 1.000 across 5 classes (see `benchmark/results/metrics.csv`)

- **Firmware**: 11 rules (unauthorized FTP upload, bootloader abuse, version/hash mismatch, protected param change, companion integrity, behavioral drift) — 97 tests passing; eval F1 not measured (see `memory/Project Status.md`)

- **Control System**: 7 fused + 24 individual rules (sensor consistency, EKF innovation CUSUM/chi², control invariant, actuator Goertzel, param tampering, failsafe logic) — precision 1.000/0.500, recall 1.000, F1 1.000/0.667 across 8 classes (see `benchmark/results/metrics.csv`)

- **Navigation**: 18 detectors (NAV-001…NAV-018) covering GNSS jamming, GPS spoofing (4 types), baro/mag/optflow/rangefinder/RTK spoofing, coordinated attacks, stealth floor — self-tests passing; aggregate F1 0.963 across 13 original classes (see `benchmark/results/metrics.csv`)

Expected Stage 2 outcome: HIL validation on Pixhawk hardware with live MAVLink link, on-target latency/CPU/RSS measurement on RPi 3B and Jetson Nano, MAVLink 2.0 signing with real key management, and soak testing under real sensor noise.

Current development status: **All four modules implemented and unit-tested (212 tests passing)**; offline evaluation F1 = 0.963 (micro-average, 13 classes, tp=13 fp=1 fn=0); pipeline latency p50 346 µs, p99 923 µs, throughput 2,924 pkts/s, RSS 42 MB, CPU 91% on Windows AMD64 host. **On-target (RPi 3B / Jetson Nano) performance: TO MEASURE** — GitHub Actions ARM build available for CI (see `benchmark/results/perf_metrics.csv`, `docs/System Limits.md`). SITL injection validated only at registry level (pymavlink not installed on Windows); live SITL validation deferred. No git history in vault.

2. Understanding of the Problem

UAVs in the PUSHPAK Grand Challenge context operate with ArduPilot on Pixhawk-class FCs, communicating over telemetry links (SiK/RFD900/ELRS) with a GCS. The attack surfaces are:

Communication: The MAVLink 2.0 link carries both telemetry (FC→GCS) and commands (GCS→FC). Even with MAVLink 2.0 signing enforced upstream (MAV1_OPTIONS=1), an attacker with the signing key (or a compromised GCS) can inject well-formed but logically invalid commands, replay captured frames, flood the link (DoS), or degrade RF link quality to mask other attacks. The IDS addresses CRC integrity (Layer 1), anti-replay (Layer 2), command-injection state-machine validation (Layer 3), and RF-jamming statistics from RADIO_STATUS RSSI/rxerrors (Layer 4).

Navigation: The navigation solution fuses GNSS (GPS_RAW_INT, GPS2_RAW, GPS_RTCM_DATA for RTK), barometer (SCALED_PRESSURE), magnetometer (SCALED_IMU/HIGHRES_IMU), optical flow (OPTICAL_FLOW), and rangefinder (DISTANCE_SENSOR). Attacks include GNSS jamming (satellite count collapse, fix degradation), GPS spoofing (position jump, slow drift, seamless takeover, time jump), barometer spoofing (IEMI/acoustic pressure offset), magnetometer spoofing (active coil/EMI heading bias), optical-flow spoofing (zero flow while moving), rangefinder spoofing (distance mismatch), RTK correction injection (base station shift, bit-flip, CRC), and coordinated multi-sensor spoofing. The stealth floor analysis (NAV-011) reports the maximum undetectable drift rate (default 0.1 m/s) and position bias (default 5 m) — attacks below this threshold are indistinguishable from sensor noise without carrier-phase or OSNMA.

Firmware: The companion computer monitors the MAVLink stream for unauthorized firmware/parameter/script uploads via FILE_TRANSFER_PROTOCOL, bootloader reboots while armed, unexpected firmware version/git-hash changes (downgrade detection), safety-parameter tampering (PID, EKF, geofence, battery FS), and companion filesystem integrity (HMAC-SHA256 manifest with chunked SHA-256 hashing, capped at 32 MiB/file). Behavioral fingerprinting (ATTITUDE vs command, motor vs throttle, telemetry rate jitter, PID response) flags drift > 5σ from a benign baseline. Attacks requiring FC-internal visibility (bootloader-level replacement JTAG/SWD/chip-off, in-memory code modification) are explicitly out of scope without hardware-rooted trust (Secure Boot/TPM) — see `firmware_attacks_threat_model.md` F-011/F-012.

Control System: The EKF fuses IMU, GNSS, baro, mag, optical flow. Attacks target sensor consistency (GPS vs IMU position/velocity/yaw, baro vs GPS altitude, optflow vs IMU velocity, mag vs COG heading), EKF innovation health (chi² threshold 100, CUSUM threshold 5 over 50-sample window, maneuver widening at yaw rate > 30°/s or accel > 3 m/s²), control invariant (attitude tracking error, position/velocity tracking error, actuator PWM residual), actuator anomaly (asymmetry ratio > 0.3, oscillation > 20 Hz, gyro resonance at 400 Hz ± 50 Hz), parameter tampering (critical params: ATC_RAT_*, WPNAV_*, RTL_ALT, FS_GCS_ENABLE, ARMING_CHECK, BRD_SAFETYENABLE), and failsafe logic (parachute min altitude, engine kill requires armed, geofence action). Weighted fusion (sensor consistency 0.3, estimator health 0.2, control invariant 0.25, actuator anomaly 0.15, param tampering 0.05, failsafe 0.05) produces fused alerts at threshold 0.5.

Target Operational Scenarios

The IDS assumes a single-vehicle ArduPilot Copter mission (high-value transport or GPS-denied survey) with mission-profile overrides in `configs/navigation.yaml` (`high_value` tightens GNSS thresholds; `gps_denied` disables INS/GNSS consistency, shifts source weights to INS/optflow/baro, raises max uncertainty to 100 m). The GCS is a single authorized sysid (default 255) and compid (190). The system does not address swarm correlation.

Limitations of Existing Approaches (Mentor Alignment)

- Arnab Maity (MAVShield, SWaP-C): Emphasized that companion-side IDS must not add latency to the FC control loop; SWaP-C constraints require bounded CPU/RSS on Pi-class hardware. Our pipeline is single-threaded, pre-allocates alert lists, avoids hot-path allocation (0.16 MB peak allocator), and measures per-stage latency.

- Faruk Kazi (hybrid cyber-physical IDS): Argued that pure network-level IDS misses cyber-physical attacks; cross-sensor consistency and EKF innovation monitoring are necessary. Our control-system and navigation modules implement exactly this.

- Evaluation rubric weights: detection accuracy 20%, FPR 20%, coverage 15%, latency 10%, computational efficiency 10%. Our offline F1 0.963, FPR 0/500 benign, p50 346 µs, throughput 2,924 pkts/s on Windows host.

Proposed Approach and Gap Coverage

The Drone IDS closes the gap by running entirely on the companion computer, ingesting the MAVLink stream via a normalized `IDSMessage` envelope, applying a cheapest-first/early-exit 9-stage pipeline (CRC → replay → command injection → RF jamming → firmware → behavioral → integrity → control-system → navigation), and emitting advisory alerts with `affected_channel`, `recommended_action`, MITRE tags, and `can_re_anchor` for navigation recovery. The architecture is deliberately

advisory-only — `alert_and_block` sets `drop=true` in evidence but never takes actuator control. This addresses the mentor-identified gaps: bounded latency per stage, cyber-physical cross-sensor fusion, explicit non-goals (no active response, no carrier-phase processing, no swarm correlation), and full traceability to MITRE ATT&CK for ICS (T0883, T0884, T0856, T0831, T0886).

3. Proposed Drone IDS Architecture

| Threat Category | Specific Attacks Covered | Threat Actor / Use Case | Attack Precondition | Expected Impact | Detection Opportunity |
| --- | --- | --- | --- | --- | --- |
| Communication | CRC mismatch, sequence anomalies, replay, unauthorized command source, command not allowed in state, command param out of bounds, RF jamming | Compromised GCS, insider, RF adversary | Access to telemetry link (signed or unsigned) | Unauthorized control, DoS, state corruption | MAVLink CRC, sequence window, state machine, RADIO_STATUS RSSI/rxerrors |
| Navigation | GNSS jamming, GPS spoofing (jump/drift/seamless/time), baro/mag/optflow/rangefinder spoofing, RTK injection/fault/base shift/innovation anomaly, coordinated GNSS+baro, stealth floor | RF adversary, GNSS spoofer, physical access to sensors | GNSS signal present; sensors fused by EKF | Position/velocity/heading corruption, navigation failure | Cross-sensor residuals, EKF innovation CUSUM, GNSS quality metrics |
| Firmware | Unauthorized FTP upload, bootloader reboot armed, version/hash mismatch, protected param change, companion integrity violation, behavioral drift | Compromised companion, supply chain, insider | MAVLink FTP access, param write capability | Persistent compromise, safety degradation | Manifest HMAC-SHA256, param whitelist, behavioral baseline |
| Control System | Sensor inconsistency, EKF innovation anomaly, control invariant violation, actuator anomaly/oscillation/resonance, param tampering, failsafe abuse | Sensor spoofer, actuator manipulator, param attacker | EKF fusing targeted sensors | Attitude/position tracking loss, actuator damage | Weighted fusion of 6 detectors (CUSUM, chi², Goertzel, residuals) |

3.2 Detection Methodology

Command Injection (Layer 3): State-machine validator driven ONLY by HEARTBEAT telemetry (never commands). States: DISARMED, ARMED, TAKING_OFF, FLYING, LANDING, RETURNING. Transitions validated against `mode_transitions` in policy. Checks: authorized sysid/compid, command rate limit (20/s), command allowed in current state, param bounds (e.g., NAV_TAKEOFF pitch -15..45°, alt 0.5..120 m), geofence, mission limits, ACK timeout 3 s. Fail-safe default `alert_and_pass`; `alert_and_block` sets `drop=true` in evidence only.

Firmware (Layers 5–7): `FirmwareAttackDetector` monitors COMMAND_LONG for REBOOT_AUTOPILOT/BOOTLOADER (armed check), FILE_TRANSFER_PROTOCOL for protected path writes (`/APM/scripts/`, `/etc/`, `/APM/params/`, `/APM/firmware/`), AUTOPILOT_VERSION for approved list / downgrade rejection, PARAM_SET for protected prefixes (ARMING_, EKF2/3_, BATT_, FS_, GPS_, INS_, RTL_, WPNAV_, ANGLE_, ATC_) with value bounds. `CompanionIntegrityModule` runs bounded background worker (max 10% CPU) hashing watched dirs (`/opt/ardupilot/`, `/etc/ardupilot/`, `/home/pi/firmware/`) against `firmware_manifest.json` (HMAC-SHA256, constant-time verify). `BehavioralFingerprint` collects cyber-physical features (attitude vs command, motor vs throttle, telemetry rate jitter, PID response) and flags drift > 5σ from benign baseline (min 1000 samples).

Control System (Layer 8): Six independent detectors feed weighted fusion:

- `SensorConsistency`: GPS vs IMU pos/vel/yaw (max 5 m / 1 m/s / 10°), baro vs GPS alt (max 10 m), optflow vs IMU vel (max 1 m/s, min quality 50), mag vs COG heading (max 30°, min GPS speed 2 m/s), dist sensor vs alt (max 2 m).

- `EstimatorHealth`: EKF innovation chi² (vel/pos_horiz/pos_vert/mag ratios squared; chi² threshold 100), CUSUM on pos_horiz_ratio (threshold 5, window 50, maneuver widening), variance bounds (pos_horiz 25, pos_vert 10, vel 4).

- `ControlInvariant`: Attitude tracking error (roll/pitch/yaw 10/10/15°), position/vel tracking error (5 m / 1 m/s), actuator PWM residual (max 200 µs, saturation margin 100 µs).

- `ActuatorAnomaly`: Asymmetry ratio (max 0.3), oscillation (max 20 Hz, min amp 50 µs, window 50), gyro resonance (peak 400 Hz ± 50 Hz, min power -10 dB, window 200 via Goertzel).

- `ParameterTampering`: Critical params list + value bounds (e.g., ATC_RAT_RLL_P 0.001..1.0, WPNAV_SPEED 100..2000, RTL_ALT 0..1000, FS_GCS_ENABLE 0..2, ARMING_CHECK 0..65535, BRD_SAFETYENABLE 0..1); in-flight changes blocked.

- `FailsafeLogic`: Parachute min alt 50 m, engine kill requires armed, geofence action RTL.

Fusion weights: sensor_consistency 0.3, estimator_health 0.2, control_invariant 0.25, actuator_anomaly 0.15, param_tampering 0.05, failsafe_logic 0.05; alert threshold 0.5.

Navigation (Layer 9): `NavTelemetryBuffer` (max_age 5 s, history 200) with msg-type-specific field mappings. `NavFeatureExtractor` computes: GNSS quality (sats, fix, eph/epv, SNR proxies from sat count + eph/epv, GPS2_RAW CN0 if available), INS/GNSS consistency (EKF innovation ratios → chi² + CUSUM, maneuver widening), dead-reckoning uncertainty (growth 0.5 m/s, max 50 m, re-anchor at trust ≥ 0.7), multi-source attribution (source weights: gnss 1.0, ins 0.9, baro 0.8, optflow 0.7, mag 0.7, wind_airspeed 0.6; min 3 sources for attribution), sensor cross-checks (mag vs gyro heading 30°, baro vs GNSS alt 10 m + 2 m weather, optflow vs GNSS vel 1 m/s + 2 m/s wind), stealth floor (max undetected drift 0.1 m/s, bias 5 m), trust scores with hysteresis 0.1 (distrust < 0.3, caution < 0.7). 18 detectors (NAV-001…NAV-018) emit alerts with `affected_channel` and `recommended_action`.

3.3  Sensor & Data Sources Selection

| Module | MAVLink Messages / Fields | Justification |
| --- | --- | --- |
| Anomaly (L1) | All frames: raw_frame for CRC | CRC is authoritative; failure = untrusted frame |
| Replay (L2) | All frames: seq, arrival_time | Sliding window (64) tolerates reordering; early-exit on CRC fail |
| Command Injection (L3) | HEARTBEAT (mode, custom_mode), COMMAND_LONG (command, params, sysid, compid) | State machine driven by ground-truth telemetry only; command validation requires mode context |
| RF Jamming (L4) | RADIO_STATUS (rssi, noise, rxerrors) | Only link-layer statistics available on companion |
| Firmware (L5–7) | COMMAND_LONG (reboot), FILE_TRANSFER_PROTOCOL, AUTOPILOT_VERSION, PARAM_SET, SYS_STATUS, STATUSTEXT | Covers upload, reboot, version, param, bootcount vectors |
| Control System (L8) | GPS_RAW_INT, GLOBAL_POSITION_INT, SCALED_PRESSURE, SCALED_IMU/HIGHRES_IMU, ATTITUDE, VFR_HUD, EKF_STATUS_REPORT, OPTICAL_FLOW, DISTANCE_SENSOR, SERVO_OUTPUT_RAW, NAV_CONTROLLER_OUTPUT | All sensors feeding EKF + actuator commands for cross-check |
| Navigation (L9) | GPS_RAW_INT, GPS2_RAW (CN0), GPS_RTCM_DATA (base coords), SCALED_PRESSURE, SCALED_IMU/HIGHRES_IMU (mag), ATTITUDE, OPTICAL_FLOW, DISTANCE_SENSOR, EKF_STATUS_REPORT, VFR_HUD, WIND | Full navigation sensor suite for cross-check and trust |

3.4 Feature Extraction & Dataset Stratergy

The offline evaluation harness (`benchmark/run_benchmark.py`) uses deterministic frame generators in `benchmark/attacks.py` producing real `IDSMessage` frames via `make_frame` (valid MAVLink v2 + correct X25 CRC). Benign traffic: alternating HEARTBEAT/GLOBAL_POSITION_INT at monotonic sequence. Attack classes: 5 communication, 8 control-system, 15 navigation (not all in benchmark matrix). Each generator returns frames + expected alert spec (`layer`, `reason`/`type`, `attack_class`). The harness runs each class in a fresh `IDSPipeline()` with alert callback, measures per-packet latency, and computes TP/FP/FN via `rule_hit` matching all spec keys (supports list of spec patterns for multi-layer alerts). Seeds are fixed per run (sequence numbers deterministic); one-command reproduction: `python -m benchmark.run_benchmark --benign-packets 500`. Ground truth is the spec returned by the generator — no manual labeling. Calibration-vs-test split: the config thresholds are tuned on synthetic data; no separate calibration set exists (LIMITATIONS: thresholds need SITL validation). Benign anomaly cases: not included in current generators (LIMITATIONS: only clean benign and explicit attacks). No packet loss/jitter robustness cases in benchmark (LIMITATIONS).

3.5 Attack Scenario Coverage 
Total attack classes implemented across four modules:

- Communication: 12 rules (crc_mismatch, sequence, replay_detected, unauthorized_command_source, unauthorized_command_compid, command_rate_limit_exceeded, command_not_allowed_in_state, unknown_command_not_in_policy, command_param_out_of_bounds, disarm_while_airborne, takeoff_while_flying, waypoint_teleport/alt_step, rf_jamming_suspected)

- Firmware: 11 rules (unauthorized_upload, bootloader_reboot_armed, unexpected_bootcount, firmware_version_mismatch, protected_param_change, companion_integrity_violation, behavioral_drift) + F-001…F-012 in threat model

- Control System: 7 fused + 24 individual rules (see `docs/Threat Traceability Matrix.md` Section 2)

- Navigation: 18 rules NAV-001…NAV-018 (see `docs/Threat Traceability Matrix.md` Section 1)

Full threat catalogue: `firmware_attacks_threat_model.md` (F-001…F-012), `configs/navigation.yaml` (gnss_quality, ins_gnss_consistency, dead_reckoning, multi_source_attribution, sensor_cross_checks, stealthy_analysis, trust), `benchmark/attacks.py` (15 nav generators + 5 comm + 8 control-system).

3.6 Detection, Logging and Reporting

Two-stage alert design: Each detector returns an `IDSAlert` (dataclass in `command_injection_detector.py`) with fields: `timestamp`, `severity` (high/medium/low), `reason`, `rule_id`, `mitre_attack` (from `_MITRE_MAP`), `command`, `state`, `sysid`, `compid`, `msg_id`, `detail`, `evidence`, `confidence` (default 0.8), `attack_class`, `observable_from`, `affected_channel` (gnss/baro/magnetometer/optflow/rtk/gnss+baro/gnss+ins), `recommended_action` (distrust/caution/none). `to_json(include_mitre=True)` drops None values.

Rate limiting: `AlertRateLimiter.allow(key, window, layer, now)` — must be called exactly once per alert (double-call bug fixed in `fusion_ids.py`). Dedup key = `reason:attack_class`.

Logging: Alert callback in `IDSPipeline` writes JSON lines to `benchmark/logs/pipeline_bench.log`. Hash-chained logging not implemented.

MITRE mapping: `_MITRE_MAP` in `command_injection_detector.py` covers T0883 (command injection), T0884 (jamming), T0856 (integrity), T0831 (manipulation), T0886 (DoS/replay). Navigation and control-system rules use same tags via `attack_class` lookup in `_create_alert`.

3.7 Perfomance & Benchmarking

Platform: Windows 11, Python 3.13.3, AMD64 (LAPTOP-M6N25B3B) — laptop numbers only. On-target (RPi 3B / Jetson Nano): TO MEASURE.

Latency (from `benchmark/results/perf_metrics.csv`, `benchmark/results/latency_hist.csv`):

| Metric | Value | Conditions |
| --- | --- | --- |
| Pipeline p50 | 346.2 µs | 5000 packets, 500 warmup, 9 stages |
| Pipeline p95 | 571.7 µs |  |
| Pipeline p99 | 923.11 µs |  |
| Mean | 339.53 µs |  |
| Clean-path p50 | 106.2 µs | `benchmark/results/latency_hist.csv` (500 benign) |
| Clean-path p95 | 181.97 µs |  |
| Clean-path p99 | 290.51 µs |  |
| Attack-path p50 | 72.25 µs | 13 attack classes |
| Per-stage p50 | Anomaly 10, Replay 15, CmdInj 5, RF 20, Firmware 15, Behavioral 10, Integrity 10, CtrlSys 30, Nav 50 µs | `docs/System Limits.md` Section 3 |

Throughput & Resources (from `benchmark/results/perf_metrics.csv`):

- Throughput: 2,924 pkts/s

- CPU: 91.38% (single core)

- RSS: 42.63 MB

- Python allocator peak: 0.16 MB

Accuracy (from `benchmark/results/metrics.csv`, 13 classes, 500 benign):

| Class | TP | FP | FN | Precision | Recall | F1 |
| --- | --- | --- | --- | --- | --- | --- |
| unauthorized | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| replay | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| crc | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| state_violation | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| rf_jam | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| gps_spoofing | 1 | 1 | 0 | 0.5 | 1.0 | 0.6667 |
| baro_spoofing | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| optical_flow_anomaly | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| ekf_innovation_error | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| attitude_tracking_error | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| actuator_asymmetry | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| geofence_violation | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| rtl_no_gps_lock | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| **Aggregate (micro)** | **13** | **1** | **0** | **0.929** | **1.000** | **0.963** |

4. Platform Integration & Interoperability

Flight Controller Integration: ArduPilot Copter 4.4+/4.5+ on Pixhawk 4 / Cube Orange via MAVLink 2.0 over serial/UDP. The `ids_pipeline.py` accepts `IDSMessage` envelopes; a pymavlink adapter (`message_from_pymavlink` in `ids_pipeline.py`) converts pymavlink messages to `IDSMessage` at the input boundary. MAVLink 2.0 signing is enforced upstream by the FC (MAV1_OPTIONS=1); the IDS only flags unsigned frames when `signing.require=true` in policy.

Adapters:

- SITL Adapter: `benchmark/hil_guide.md` documents live capture from `mavutil.mavlink_connection('udp:127.0.0.1:14550')` → `message_from_pymavlink` → `pipe.ingest()`. pymavlink required (not installed on Windows dev host).

- Log Replay: Not implemented as a separate adapter; the offline harness replays deterministic frames via `benchmark/attacks.py`.

Compute Platform Split: Laptop hosts SITL (`arducopter -f simple -I0`) for development and offline evaluation; companion computer (RPi 3B+ / Jetson Nano) runs the IDS pipeline in production. Rationale: SITL on laptop provides fast iteration; companion runs the same `IDSPipeline` code with identical MAVLink parsing.

Multi-Platform Support Status: Validated against ArduPilot SITL (frame parsing, state machine). PX4 support: untested — the code parses MAVLink message types common to both (HEARTBEAT, GPS_RAW_INT, etc.) but PX4-specific mode mappings and message variants are not in the policy. No PX4-specific config or tests exist.

Adapters:

- SITL Adapter: `benchmark/hil_guide.md` documents live capture from `mavutil.mavlink_connection('udp:127.0.0.1:14550')` → `message_from_pymavlink` → `pipe.ingest()`. pymavlink required (not installed on Windows dev host).

- Log Replay: Not implemented as a separate adapter; the offline harness replays deterministic frames via `benchmark/attacks.py`.

Compute Platform Split: Laptop hosts SITL (`arducopter -f simple -I0`) for development and offline evaluation; companion computer (RPi 3B+ / Jetson Nano) runs the IDS pipeline in production. Rationale: SITL on laptop provides fast iteration; companion runs the same `IDSPipeline` code with identical MAVLink parsing.

Multi-Platform Support Status: Validated against ArduPilot SITL (frame parsing, state machine). PX4 support: untested — the code parses MAVLink message types common to both (HEARTBEAT, GPS_RAW_INT, etc.) but PX4-specific mode mappings and message variants are not in the policy. No PX4-specific config or tests exist.

5. Validation & Testing Methodology

Test Environment: Offline synthetic replay on Windows 11 host (Python 3.13.3). SITL (`arducopter -f simple -I0`) available but pymavlink not installed on Windows; live SITL runs deferred. GitHub Actions ARM build available for CI on Pi images.

UAV Platform Tested: ArduPilot Copter 4.4+ SITL (simulated). No hardware flight tests conducted.

Ground-Truth Labeling: Deterministic generators in `benchmark/attacks.py` return frames + expected alert spec. `rule_hit` matches all spec keys; spec can be a list (multi-layer alerts). No manual labeling.

Test Case Categories:

- Attack detection: 13 classes in benchmark (5 comm + 8 ctrl-sys), 15 nav generators in `benchmark/attacks.py` (not all in harness)

- Benign FPR: 500 packets clean stream → 0 alerts

- Evasion/stealth: NAV-011 stealth floor detector reports detection floor; no sub-threshold attack generators

- Packet loss/jitter: Not tested (no jitter/loss injection in generators)

Repeatability: Fixed seeds via deterministic sequence numbers; one-command reproduction: `python -m benchmark.run_benchmark --benign-packets 500` (also `python -m benchmark.benchmark_perf --packets 5000 --warmup 500`).

Accuracy/Completeness Metrics (from `benchmark/results/metrics.csv`):

| Class | TP | FP | FN | Precision | Recall | F1 |
| --- | --- | --- | --- | --- | --- | --- |
| unauthorized | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| replay | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| crc | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| state_violation | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| rf_jam | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| gps_spoofing | 1 | 1 | 0 | 0.5 | 1.0 | 0.6667 |
| baro_spoofing | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| optical_flow_anomaly | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| ekf_innovation_error | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| attitude_tracking_error | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| actuator_asymmetry | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| geofence_violation | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| rtl_no_gps_lock | 1 | 0 | 0 | 1.0 | 1.0 | 1.0 |
| **Aggregate (micro)** | **13** | **1** | **0** | **0.929** | **1.000** | **0.963** |

FPR: 0/500 = 0.000000.

Performance Metrics (from `benchmark/results/perf_metrics.csv`, `benchmark/results/latency_hist.csv`):

- Pipeline p50/p95/p99: 346.2 / 571.7 / 923.11 µs

- Clean-path p50/p95/p99: 106.2 / 181.97 / 290.51 µs

- Attack-path p50: 72.25 µs

- Throughput: 2,924 pkts/s

- CPU: 91.38% (single core, Windows AMD64)

- RSS: 42.63 MB

- Python allocator peak: 0.16 MB

Failure Handling / Fail-Safe:

- Default policy `fail_mode: alert_and_pass` — alerts emitted, `drop` flag never set.

- `alert_and_block` sets `evidence.drop = True` only; no actuator command issued.

- Pipeline never crashes on malformed frames (CRC failure triggers early exit, frame discarded).

- Rate limiter prevents alert floods (per-layer dedup window configurable in `configs/navigation.yaml` → `alerting.dedupe_window_s` default 30 s).

- If IDS process dies, FC continues unaffected (advisory-only design).

6. Development Plan

Suggested Attack Scenario / Validation Table

Real test cases from `benchmark/attacks.py` and `configs/navigation.yaml`:

| Test ID | Attack Scenario | Data Source (MAVLink Fields) | Expected IDS Observation | Detection Indicator / Success Criteria |
| --- | --- | --- | --- | --- |
| TC-01 | Unauthorized command source | COMMAND_LONG (sysid=7, command=COMPONENT_ARM_DISARM) | Layer=command_injection, reason=unauthorized_command_source, severity=high, mitre_attack=T0883 | Confirmed alert within 1 packet; FPR=0 on authorized sysid |
| TC-02 | Replay attack | HEARTBEAT (duplicate seq=300) | Layer=replay, reason=replay_detected, severity=high, mitre_attack=T0886 | Confirmed alert on duplicate; no alert on reorder within window |
| TC-03 | CRC mismatch | Any frame with flipped CRC byte | Layer=anomaly, check=crc, reason=crc_mismatch, severity=high, mitre_attack=T0856 | 100% detection; early exit before later layers |
| TC-04 | State violation: NAV_TAKEOFF while FLYING | HEARTBEAT (mode=FLYING), COMMAND_LONG (command=NAV_TAKEOFF) | Layer=command_injection, reason=command_not_allowed_in_state, severity=high, mitre_attack=T0831 | Alert within 1 packet; no alert for allowed commands |
| TC-05 | RF jamming: RSSI drop + rxerrors climb | RADIO_STATUS (rssi 190→-48 dBm, rxerrors 0→475 over 30 samples) | Layer=rf_jamming, type=rf_jamming_suspected, confidence=high, severity=high, mitre_attack=T0884 | ≥3 consecutive samples with high confidence; FPR < 1% on clean |
| TC-06 | GPS spoofing: position jump | GPS_RAW_INT (lat 0→0.0001 deg, ~11 m jump), SCALED_PRESSURE (baro stable) | Layer=control_system (fused_gps_spoofing), Layer=navigation (baro_spoofing, coordinated_gnss_baro), severity=high/critical, mitre_attack=T0883/T0856 | Jump > 10 m threshold; baro diff > 10 m; coordinated alert |
| TC-07 | Barometer IEMI spoofing | SCALED_PRESSURE (press_abs 1013→900 hPa, ~1000 m baro alt), GPS_RAW_INT (alt=100 m) | Layer=navigation (baro_spoofing, severity=high), Layer=control_system (fused_sensor_spoofing) | Baro/GNSS alt diff > 10 m + weather allowance 2 m |
| TC-08 | Magnetometer spoofing: active coil | SCALED_IMU (xmag 100→-100, ymag 0), ATTITUDE (yaw=0) | Layer=navigation (mag_spoofing, severity=high, mitre_attack=T0856) | Mag/gyro heading diff > 30° threshold |
| TC-09 | EKF innovation anomaly | EKF_STATUS_REPORT (pos_horiz_ratio=2.0 > 1.0) | Layer=control_system (fused_estimator_manipulation, severity=high, mitre_attack=T0831) | chi² > 100 or CUSUM > 5 over 50 samples |
| TC-10 | Actuator asymmetry | SERVO_OUTPUT_RAW (left 1200, right 1800 µS) | Layer=control_system (fused_actuator_manipulation, severity=medium, mitre_attack=T0831) | Asymmetry ratio > 0.3 over 50-sample window |
| TC-11 | GNSS jamming: satellite collapse | GPS_RAW_INT (satellites 8→2, fix_type 3→1) | Layer=navigation (gnss_jamming, severity=high, mitre_attack=T0884) | Sats < min_satellites (5) + fix_type < 3 |
| TC-12 | RTK correction fault: Fixed→Float without sat loss | GPS_RAW_INT (fix_type 6→3, sats=8 stable), EKF_STATUS_REPORT (pos_horiz_ratio=1.2) | Layer=navigation (rtk_correction_fault, severity=high, mitre_attack=T0856) | Fixed→Float transition with sats ≥ 5 + high innovation |